VOXA

Hardware Wiring & Connection Reference

ESP32-S3 Pinout Documentation for Display, Touch, Audio, Storage, and Power Peripherals

Document Version 1.0

August 10, 2026

# Overview

This document lists the complete physical wiring for every peripheral connected to the VOXA device's ESP32-S3 microcontroller. It covers the display, capacitive touch panel, audio amplifier, MEMS microphone, MicroSD card module, and battery monitoring circuit. All GPIO assignments listed reflect the current firmware pin configuration.

| # | Peripheral | Interface | Pins Used |
| --- | --- | --- | --- |
| 1 | ST7789 240x320 SPI Display | SPI | 7 |
| 2 | CST328 Capacitive Touch Screen | I2C | 4 |
| 3 | MAX98357A I2S Audio Amplifier | I2S | 5 |
| 4 | INMP441 I2S MEMS Microphone | I2S | 4 |
| 5 | MicroSD Card Module | SPI | 4 |
| 6 | Battery Monitoring | ADC | 1 |

# Peripheral Connections

# 1. ST7789 240×320 SPI Display

| Module Pin | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| VCC | 3.3V | Power supply |
| GND | GND | Ground |
| SCL / SCLK | GPIO 12 | SPI clock |
| SDA / MOSI | GPIO 11 | SPI data (master out) |
| CS | GPIO 10 | Chip select |
| DC / RS | GPIO 9 | Data / command select |
| RES / RST | GPIO 14 | Reset |
| BLK / BL | GPIO 3 | Backlight control (PWM) |

# 2. CST328 Capacitive Touch Screen (I2C)

| Module Pin | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| VCC | 3.3V | Power supply |
| GND | GND | Ground |
| SDA | GPIO 8 | I2C data |
| SCL | GPIO 18 | I2C clock |
| RST | GPIO 17 | Reset |
| INT | GPIO 16 | Interrupt |

# 3. MAX98357A I2S Mono Audio Amplifier (Speaker)

| Module Pin | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| VIN / VCC | 5V (or 3.3V) | Power supply |
| GND | GND | Ground |
| BCLK | GPIO 40 | I2S bit clock |
| LRC / WS | GPIO 41 | I2S word select (L/R clock) |
| DIN | GPIO 39 | I2S serial data in |
| GAIN | GND / Float | GND = 12 dB gain · Float = 9 dB gain |
| SD (Shutdown) | 3.3V / Float | Active HIGH — tie high or leave floating to enable |

Caution: keep LRC and DIN wires physically separated. Even brief contact between these two lines corrupts both the audio data and channel timing, resulting in no sound or static output.

# 4. INMP441 I2S MEMS Microphone

| Module Pin | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| VDD / VCC | GPIO 42 | Software-controlled power pin, driven HIGH at boot |
| GND | GND | Ground |
| SCK | GPIO 4 | I2S bit clock |
| WS | GPIO 5 | I2S word select |
| SD | GPIO 7 | I2S serial data out |
| L/R | GND | Selects left I2S channel |

# 5. MicroSD Card Module (SPI)

| Module Pin | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| VCC | 5V (or 3.3V) | Depends on onboard regulator |
| GND | GND | Ground |
| CS | GPIO 1 | Chip select |
| MOSI | GPIO 2 | SPI data (master out) |
| MISO | GPIO 13 | SPI data (master in) |
| SCK | GPIO 21 | SPI clock |

# 6. Battery Monitoring (ADC)

| Module Pin | ESP32-S3 Pin | Notes |
| --- | --- | --- |
| BAT_ADC | GPIO 6 (ADC1_CH5) | Via resistor voltage divider from LiPo / 18650 positive terminal |